## Fluorescent Lamp Operation:

- Switch On.

- Starter Glows: The line voltage causes a glow discharge across the small gap in the starter, ionizing its internal gas.

- Strip Heats/Bends: This heat causes the bimetallic strip inside the starter to bend and close the circuit.

- High Current/Pre-heat: A relatively high current flows through the circuit (ballast, filaments, then closed starter).

  - Filaments Emit: The current rapidly heats the filaments, causing them to release an electron cloud (thermionic emission).

  - Ballast Stores Max Energy: The high current flow in this closed path simultaneously builds the ballast's maximum magnetic field (stored energy).

  - Starter Stops Glowing: The closed strip shorts out the voltage needed for the glow discharge, so the starter light goes out.

- Strip Cools/Opens: The strip cools rapidly and snaps back, opening the circuit.

- Current Interrupted.

- Inductive Kick: The ballast's magnetic field suddenly collapses, generating a very high voltage pulse (the inductive kick) across the tube.

- Arc Strikes: This high voltage forces the electron cloud to rush across the tube, ionizing the mercury and inert gas to create a plasma arc (the conductive path).

- Light Generation (Continuous):

  - The arc current continuously excites mercury atoms.

  - Mercury atoms emit UV light.

  - UV light hits the phosphor coating, which emits visible light (fluorescence).

- Continuous Current Limit: The ballast remains in the circuit, using its inductive reactance to stabilize the arc current and prevent the tube from self-destructing.

- Starter Inactive: The low voltage across the running tube is insufficient to restart the glow discharge, so the starter remains open.